Assessment of CO2 Abatement Potential of Heat Pumps using Pinch Analysis for the Swiss Chocolate Industry

Navdeep Bhadbhadea*, Benjamin H. Y. Ongb, Donald G. Olsen a, Beat Wellig a Martin K. Patela

a Chair for Energy Efficiency, Department F.-A. Forel for Environment and Aquatic Sciences, Institute for Environmental Sciences (ISE), University of Geneva, Switzerland

b Lucerne University of Applied Sciences and Arts, Competence Center Thermal Energy Systems and Process Engineering, Technikumstrasse 21, 6048, Horw, Switzerland

Appendix 1: Cost-effectiveness analysis

Clvl=ANF×NPVES 										Eq. A1

Where,

Clvl, = Levelized costs of saved energy by HP integration (CHF/kWh)

ANF = Annuity factor calculated by equation (A2)

NPV = Net present value of an EEM calculated by equation (A3) (CHF)

ES = Final energy saving achieved by HP integration (MWh)

ANF=(1+r)L×r(1+r)L-1 										Eq. A2

And

NPV=t=trLCFtr×(1+r)-tr+t								Eq. A3

Where,

CFtr = Annual cash flow for year t calculated by equation (A4) (CHF)

r = Real discount rate (%)

L = Technological lifetime of HP (in years)

tr = running year (e.g., 2020)

t = year of implementation

CFtr=I+O&M-B 										Eq. A4

Where,

I = Initial investment for of HP integration

O&M = Annual operation and maintenance cost of HP

B = Annual net benefits generated by implementing HP integration calculated by equation (A5)

B=FS×P-(ELS×Pe)+(CA×PCO2)  							Eq. A5

Where,

ELS = Electricity consumed by HP integration estimated using COPreal

FS = Saving achieved for fuel by HP integration

P = Price of fuel (Since thermal demand is dominated by natural gas, fuel saved is assumed to be natural gas for all establishments)

Pe = Price of electricity e

PCO2 = CO2 levy

CA = CO2 abatement potential of HP integration calculated by equation A6

CA=FS×EF  										Eq. A6

Where,

CA = carbon dioxide abatement achieved by HP integration (t-CO2/year)

FS = Fuel savings estimated based on HP heating capacity

EF  = CO2 emission factor for fuel (t-CO2/kWh)

Appendix 2 Electricity and natural gas prices for all industrial establishment categories and cantons in Switzerland

Table A1 presents the electricity and natural gas price paid by the industrial establishments of various size categories across different cantons. The economic feasibility analysis presented in section 3.3 is based on the ratio of average electricity price and average natural gas price for each category across the cantons. The sensitivity analysis presented in subsequent section 3.4 on the other hand, is based on the ratio of the energy prices for the canton with maximum electricity price and minimum electricity price identified from the following table.

Table A1. Natural gas and electricity prices for various sizes and cantons for the Swiss industry (Electricity prices (ElCom, 2023), Natural gas prices (DFE, 2023) )

| Canton | Natural gas prices (CHF/kWh) |  |  | Electricity (CHF/kWh) |  |  | Ratio (Electricity/Gas) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Small (Type 8) | Medium (Type 9) | Large (Type 10) | Small (C3) | Medium (C5) | Large (C7) | Small (C3) | Medium (C5) | Large (C7) |
| Aargau | 0.16 | 0.16 | 0.16 | 0.17 | 0.13 | 0.12 | 1.06 | 0.81 | 0.75 |
| Appenzell | 0.18 | 0.13 | 0.12 | 0.16 | 0.14 | 0.13 | 0.89 | 1.08 | 1.08 |
| Bern | 0.16 | 0.16 | 0.16 | 0.21 | 0.18 | 0.16 | 1.31 | 1.13 | 1.00 |
| Freiburg | 0.1 | 0.12 | 0.12 | 0.19 | 0.15 | 0.14 | 1.90 | 1.25 | 1.17 |
| Geneva | 0.097 | 0.097 |  | 0.19 |  | 0.14 | 1.96 | - | - |
| Glarus | 0.17 | 0.1 | 0.1 | 0.18 | 0.14 | 0.13 | 1.06 | 1.40 | 1.30 |
| Graubunden | 0.1 | 0.1 |  | 0.2 | 0.18 | 0.17 | 2.00 | 1.80 | - |
| Luzern | 0.13 |  |  | 0.18 | 0.14 | 0.13 | 1.38 | - | - |
| St. Gallen | 0.13 | 0.13 | 0.12 | 0.17 | 0.14 | 0.13 | 1.31 | 1.08 | 1.08 |
| Schaffhausen | 0.06 |  |  | 0.17 | 0.14 | 0.12 | 2.83 | - | - |
| Solothurn | 0.17 | 0.17 | 0.17 | 0.19 | 0.16 | 0.15 | 1.12 | 0.94 | 0.88 |
| Schwyz | 0.19 | 0.21 | 0.21 | 0.18 | 0.15 | 0.14 | 0.95 | 0.71 | 0.67 |
| Thurgau | 0.13 | 0.08 | 0.08 | 0.18 | 0.15 | 0.14 | 1.38 | 1.88 | 1.75 |
| Ticino | 0.1 | 0.07 | 0.07 |  |  |  | - | - | - |
| Vaud | 0.1 | 0.13 | 0.12 | 0.17 |  | 0.14 | 1.70 | - | 1.17 |
| Valais | 0.07 | 0.101 | 0.1 | 0.18 | 0.15 | 0.14 | 2.57 | 1.49 | 1.40 |
| Zug | 0.12 |  |  | 0.18 | 0.16 | 0.13 | 1.50 | - | - |
| Zurich | 0.15 | 0.15 | 0.15 | 0.16 | 0.12 | 0.12 | 1.07 | 0.80 | 0.80 |
| Swiss average | 0.13 | 0.13 | 0.13 | 0.18 | 0.15 | 0.14 | 1.40 | 1.17 | 1.06 |

Appendix 3: Representative grand composite curves for company size categories

| (A) | (B) |
| --- | --- |
| (C) | (D) |

Figure A1. Representative GCC and possible heat sinks (red lines) and sources (green lines) for chocolate production in Switzerland: (A) category A1 (1000 tonnes/year), (B) category A2 (10,000 tonnes/year), (C) category A3 (20,000 tonnes/year), (D) category A4 (30,000 tonnes/year) (Pinch point at 38°C; global ∆Tmin = 10°C)

Figure A2. Representative grand composite curve for production route 2 (nibs roasting)

Appendix 4 Scaling factors and average values

Table A2. Scaling factors and other used of the Swiss chocolate industry

|  | Size category 1 | Size category 2 | Size category 3 | Size category 4 |
| --- | --- | --- | --- | --- |
| Operating hours | 3600 | 4800 | 6000 | 6000 |
| Scaling factor | 1.44 | 2.01 | 2.01 | 2.4 |
| Average production (kt) | 850 | 8922 | 18,230 | 28,458 |
| Average fuel demand (MWh/year) | 258 | 2576 | 8030 | 25,238 |
| Average electricity demand (MWh/year) | 185 | 2055 | 6407 | 20,137 |

Appendix 5: Detailed levelized cost estimates for all heat pumps

Table A3. Annual potentials and Levelized costs of saved energy and CO2 abatement for the establishment of category A1 (1000 tonne/year; results by row are mutually exclusive)

|  |  | Levelized cost |  |
| --- | --- | --- | --- |
| Heat pump model | Arrangement | Saved energy (CHF/kWh saved) | CO2 abatement (CHF/kWh abated) |
| SHP4 | AR1 | -0.09 | -284 |
| SHP2 | AR1 | 0.01 | 29 |
| SHP1 | AR1 | 0.02 | 83 |
| SHP3 | AR1 | 0.32 | 307 |

Table A4. Levelized costs and annual saving potentials for the establishments of size category 2 (10,000 tonne/year, DR = 21%)

|  |  | Levelized cost |  |
| --- | --- | --- | --- |
| Heat pump model | Arrangement | Saved energy (CHF/kWh saved) | CO2 abatement (CHF/kWh abated) |
| SHP6 | AR1/AR3 | -0.04 | -146 |
| SHP12 | AR1/AR3 | -0.05 | -155 |
| SHP18 | AR1/AR3 | -0.10 | -412 |
| SHP8 | AR1/AR3 | -0.11 | -404 |
| SHP14 | AR1/AR3 | -0.11 | -373 |
| SHP10 | AR1/AR3 | -0.11 | -418 |
| SHP11 | AR1/AR3 | -0.12 | -428 |
| SHP15 | AR1/AR3 | -0.12 | -386 |
| SHP21 | AR1/AR3 | -0.12 | -457 |
| SHP20 | AR1/AR3 | -0.12 | -483 |
| SHP7 | AR1/AR3 | -0.12 | -400 |
| SHP9 | AR1/AR3 | -0.12 | -461 |
| SHP19 | AR1/AR3 | -0.12 | -446 |
| SHP13 | AR1/AR3 | -0.12 | -473 |
| SHP16 | AR1/AR3 | -0.13 | -479 |
| SHP17 | AR1/AR3 | -0.13 | -547 |
| HTHP1 | AR2/AR4 | 0.11 | 192 |
| HTHP2 | AR2/AR4 | 0.08 | 100 |
| HTHP3 | AR2/AR4 | 0.1 | 90 |

Table A5. Levelized costs and annual saving potentials for the establishments of category 3 (20,000 tonne/year, DR = 21%)

|  |  | Levelized cost |  |
| --- | --- | --- | --- |
| Heat pump model | Arrangement | Saved energy (CHF/kWh saved) | CO2 abatement (CHF/kWh abated) |
| SHP23 | AR1/AR3 | -0.11 | -394 |
| SHP24 | AR1/AR3 | -0.14 | -543 |
| HTHP2 | AR2/AR4 | 0.06 | 46 |

Table A6. Levelized costs and annual saving potentials for the establishments of category 4 (30,000 tonne/year, DR = 21%)

|  |  | Levelized cost |  |
| --- | --- | --- | --- |
| Heat pump model | Arrangement | Saved energy (CHF/kWh saved) | CO2 abatement (CHF/kWh abated) |
| SHP24 | AR1/AR3 | -0.13 | -523 |
| SHP25 | AR1/AR3 | -0.05 | -31 |
| HTHP3 | AR2/AR4 | 0.07 | 43 |

Appendix 6 Data for product-specific energy demand profile for chocolate production in Switzerland

| Process | Process stream | Tin (°C) | Tout (°C) | ΔH (MWh) |
| --- | --- | --- | --- | --- |
| Roasting | zone 1 hot air in | 20 | 145 | 12,380 |
| Roasting | zone 1 hot air out | 30 | 20 | 4,767 |
| Roasting | zone 2 hot air in | 20 | 155 | 14,688 |
| Roasting | zone 2 hot air out | 46 | 20 | 7,064 |
| Roasting | zone 3 hot air in | 20 | 160 | 19,181 |
| Roasting | zone 3 hot air out | 110 | 20 | 12,462 |
| Blending | pressb liq in | 43 | 50 | 100 |
| Blending | pressb torteu out | 45 | 20 | 184 |
| Blending | pressh in | 20 | 50 | 110 |
| Blending | pressh tourteau | 50 | 20 | 223 |
| CIP | Lauge | 80 | 80.1 | 600 |
| CIP | Saure | 60 | 60.1 | 208 |
| CIP | Spulwasser | 10 | 40 | 496 |
| Compressor | WRG | 70 | 16.1 | 20,416 |
| Conching | cfcaill phase 1 in | 40 | 55 | 1,252 |
| Conching | cffonc phase 1 in | 35 | 80 | 1,688 |
| Conching | cbmag phase 1 in | 35 | 42 | 93 |
| Conching | cbfoncee phase 1 in | 35 | 42 | 148 |
| Conching | cfcaill refroid in | 55 | 20 | 18,440 |
| Conching | cffonc refroid in | 80 | 20 | 23,709 |
| Conching | cbmag refroid in | 42 | 20 | 1,159 |
| Conching | cbfoncee refroid in | 42 | 20 | 7,244 |
| Debacterization | Debac_cocoa_in | 25.0 | 80.0 | 3,396 |
| IR | Combustion Air_s | 30 | 70 | 2,807 |
| IR | Combustion Air_w | 20 | 70 | 3,506 |
| IR | Exhaust after cyclone | 150.0 | 25.0 | 8470.6 |
| Mixer | Jacketed vessel | 164.0 | 165.0 | 9792.8 |
| Moulding | lca mel in | 30 | 35 | 398 |
| Moulding | lca mel latmg | 35 | 36 | 668 |
| Moulding | lca mel mgfus | 36 | 39 | 292 |
| Moulding | lca mel out | 39 | 41 | 220 |
| Moulding | lca prebroy in | 41 | 48 | 624 |
| Moulding | lca prebroy out | 48 | 49 | 96 |
| Moulding | lfonc mel in | 30 | 42 | 782 |
| Moulding | lfonc mel out | 42 | 43 | 74 |
| Moulding | lfonc prebroy in | 43 | 54 | 795 |
| Moulding | lfonc prebroy out | 54.0 | 55.0 | 87 |
| Refrigeration | Industrial water | 35.0 | 25.0 | 73,110 |
| Refrigeration | Industrial water (Desuperheater) | 60.0 | 25.0 | 12,332 |
| Tamper | tankboyeuse in | 45 | 46 | 5,499 |
| Tamper | tankmasses in | 42 | 43 | 8,651 |
| Tamper | tankmass suite in | 42 | 43 | 737 |
| Tamper | tankliqueur in | 45 | 46 | 191 |
| Tamper | tankmout noisam in | 47 | 48 | 438 |
| Tamper | tankhuil noisam in | 43 | 44 | 246 |
| Tamper | tankbeur camion in | 50 | 51 | 1,098 |
| Tamper | tankbeur in | 45 | 46 | 550 |
| Tamper | tankconf in | 42 | 43 | 273 |
| Tornado Roaster | Vapour Exhaust | 120 | 30 | 626 |
| Tornado Roaster | FG Exhaust_s | 300 | 25 | 2,523 |
| Tornado Roaster | FG Exhaust_w | 300 | 25 | 2,523 |
| Tornado Roaster | Combustion Air_s | 30 | 70 | 216 |
| Tornado Roaster | Combustion Air_w | 20.0 | 70.0 | 269 |
| Tornado Roaster | Exhaust_Nibscooler_s | 60 | 25 | 2,096 |
| Tornado Roaster | Exhaust_Nibscooler_w | 60 | 25 | 2,096 |
| Tornado Roaster | Supply air_nibscooler_s | 30 | 14 | 958 |
| Tornado Roaster | Beans | 70 | 120 | 18,371 |
| Tracing circuits | Tracing water | 25 | 55 | 43,186 |

Appendix 7 Process equipment data

| Equipment | Source |
| --- | --- |
| Roaster | (Buhler, 2022b), ((HDM), 2023), (Germán Giner Santonja, 2019) |
| Dryer | ((RDW), 2024), (Buhler, 2022a), (Germán Giner Santonja, 2019) |
| Refrigeration | (Germán Giner Santonja, 2019), (GEA, 2022), (Controls, 2022) |

References:

(HDM), H. D. M. G. 2023. CONTINUOUS COCOA BEAN ROASTER FRV [Online]. https://www.h-d-m.com/en/products/cocoa-processing/roasting/continuous-cocoa-bean-roaster-frv/.  [Accessed].

(RDW), R. D. W. 2024. Continuous Bean Dryer [Online]. https://duyviswiener.com/equipment/cocoa-processing/continuous-bean-dryer/.  [Accessed].

BUHLER. 2022a. Infrared pre-treatment system [Online]. https://www.buhlergroup.com/global/en/products/barth_infra-red_dryer.html.  [Accessed].

BUHLER. 2022b. Tornado roasting system [Online]. https://www.buhlergroup.com/global/en/products/barth_tornado_roastingsystem.html.  [Accessed].

CONTROLS, J. 2022. Chillers based on Sabroe core technologies [Online]. https://www.johnsoncontrols.com/en_th/industrial-refrigeration/chillers-and-heat-pumps/chillpac.  [Accessed].

DFE 2023. Surveillant des prix relatif aux prix du gaz naturel. In: L’ÉCONOMIE, D. F. D. (ed.). https://gaspreise.preisueberwacher.ch/web/index.asp?l=1.

ELCOM 2023. Prix de l’électricité en Suisse. https://www.strompreis.elcom.admin.ch: ElCom.

GEA. 2022. GEA chiller system [Online]. https://www.gea.com/en/products/chillers/.  [Accessed].

GERMÁN GINER SANTONJA, P. K., KRISTINE RAUNKJÆR STUBDRUP, THOMAS BRINKMANN, SERGE ROUDIER 2019. Best Available Techniques (BAT) Reference Document for the Food, Drink and Milk Industries.